Critical Assessment of the Implementation

For the evaluation, I used the modified Renode setup with four nodes in a line topology. The UART outputs of the four nodes were exposed through TCP sockets and collected with a Python script. The script recorded send and receive events based on the sequence number and message type and then calculated the average propagation latency and reliability per node. I evaluated the implementation once with the range-based wireless medium without loss and once with the lossy wireless medium.

Latency

The measured propagation latency increases with the number of hops. In the no-loss scenario, the average latency was approximately:

node1: about 2560 ms
node2: about 1080 ms
node3: about 460 ms

In the lossy scenario, the measured values were approximately:

node1: about 2300 ms
node2: about 1350 ms
node3: about 970 ms

At first sight, the order seems counter-intuitive, because one would expect nodes that are farther away from the initiator to have a higher latency. This result is caused by the way the implementation and the evaluation script work. The same packet is forwarded by multiple nodes, and the script uses the first observed transmission as the reference point. Additionally, BLE advertising is asynchronous, and each forwarding step includes a delay due to the advertising duration, scanning, sleeping, and logging. Therefore, the measured latency is not only the pure wireless propagation delay, but also includes implementation-specific delays.

In general, the expected behavior is that every additional hop adds additional delay. Since my implementation advertises for approximately 500 ms per forwarding event, a roughly linear increase in latency with the number of forwarding steps is expected. The measurements confirm that the forwarding process introduces significant delay, especially when packets have to be propagated over multiple hops.

Reliability

The no-loss scenario did not reach 100 % reliability. The measured reliability was about 90 % for node1, node2, and node3, while node4 did not appear in the result. In the lossy scenario, reliability decreased further:

node1: about 80 %
node2: about 90 %
node3: about 80 %
node4: 0 % / not observed

This shows that the implementation is not perfectly reliable, even when the simulated wireless medium itself does not introduce packet loss. One reason is that BLE advertising is connectionless. Packets are broadcast without acknowledgements, and receivers may miss advertisements if they are not scanning at the right time or if they are currently advertising themselves. Another reason is that my implementation does not use retransmissions. Each packet is only forwarded once by each node. Therefore, if one forwarding step fails, all nodes behind that hop will not receive the packet.

This effect is especially important in a line topology. A packet loss near the beginning of the line is more severe than a packet loss at the last hop, because all following nodes depend on the previous node to forward the message. In the lossy medium, this explains why the last node may lose complete packets and sometimes never receive a command.

The fact that node4 is missing in the plots indicates an additional evaluation or implementation issue. Possible reasons are that node4 was not correctly connected to the Python script, that its UART output did not match the expected regular expression, or that it never received and forwarded matching packets. This should be checked by inspecting the raw UART output of term_4.

Scalability

The implementation does not scale very well to larger networks. It uses a simple flooding approach: every node that receives a new packet rebroadcasts it until the TTL expires. This is acceptable for four nodes in a line topology, but in larger networks it can lead to several problems.

First, the amount of traffic increases quickly because many nodes may rebroadcast the same packet. This can cause collisions and increase the probability that advertisements are missed. Second, the latency grows with the number of hops, because each hop adds advertising and scanning delay. Third, the sequence number is only 8 bit wide, so it wraps around after 256 messages. Since duplicate detection is only based on the sequence number, old and new messages may be confused after wrap-around.

A further limitation is the duplicate detection in the code. Currently, only the sequence number is stored. A better design would include at least the sender ID and the sequence number, for example (source_node_id, sequence_number). Otherwise, two different nodes using the same sequence number could incorrectly be treated as duplicates.

Topology

The current design was tested in a line topology, but it is not strictly limited to it. Since the implementation uses TTL-based flooding, it could also work in simple mesh-like topologies. However, for a real mesh network, several improvements would be necessary.

The packets should contain a source node ID, a larger sequence number, and possibly a previous-hop field. Duplicate detection should be based on source and sequence number, not only on sequence number. For better reliability, acknowledgements and retransmissions could be added. For better scalability, controlled flooding or routing could be used instead of rebroadcasting every packet blindly.

Possible Improvements

Several improvements could make the implementation more robust:

1. Add acknowledgements and retransmissions.
2. Use a larger sequence number, e.g. 16 bit or 32 bit.
3. Include a unique sender or source node ID in every packet.
4. Improve duplicate detection using (source_id, sequence_number).
5. Advertise each packet multiple times instead of only once.
6. Add randomized forwarding delays to reduce collisions.
7. Measure and log timestamps directly on the nodes for more accurate latency measurements.
8. Fix the duplicate detection loop from <= counter to < counter.

The current implementation is therefore a working proof of concept for multi-hop propagation of button events, but it is not yet a robust or scalable network protocol. It demonstrates the basic idea of flooding a command through a small network, but the evaluation shows that connectionless BLE advertising without acknowledgements is vulnerable to packet loss and timing issues.